UniFi Network Setup Procedure

UCG Ultra  |  USW-48-PoE  |  U7-Lite WAPs

Site Details

| Site Name |  |
| --- | --- |
| Site Address |  |
| Date |  |
| Technician Name |  |
| UCG Ultra Serial No. |  |
| Job Number |  |

Network Configuration Reference

| Gateway IP | 192.168.10.1 |
| --- | --- |
| Subnet Mask | 255.255.255.0 (/24) |
| DHCP Range | 192.168.10.100 – 192.168.10.254 |
| Primary DNS | 1.1.1.1 (Cloudflare) |
| Secondary DNS | 8.8.8.8 (Google) |

VLAN Assignments

| VLAN ID | Name | Subnet | Purpose |
| --- | --- | --- | --- |
| 10 | Staff / Management | 192.168.10.0/24 | Staff devices, admin |
| 20 | Equipment / IoT | 192.168.20.0/24 | Cardio machines, IoT |
| 30 | Guest | 192.168.30.0/24 | Member WiFi, isolated |

WiFi Networks

| SSID | Security | VLAN | Password |
| --- | --- | --- | --- |
| [Site] Staff | WPA3/WPA2 | 10 |  |
| [Site] Cardio | WPA2 | 20 |  |
| [Site] Guests | WPA2 | 30 |  |

Device Static IP Assignments

| # | Device | Model | IP Address | MAC Address |
| --- | --- | --- | --- | --- |
| 1 | Gateway | UCG Ultra | 192.168.10.1 |  |
| 2 | Switch | USW-48-PoE | 192.168.10.2 |  |
| 3 | WAP 1 | U7-Lite | 192.168.10.3 |  |
| 4 | WAP 2 | U7-Lite | 192.168.10.4 |  |

Phase 1 — Physical Installation & Rack Build

☐  Mount patch panel in rack (top position)

☐  Mount USW-48-PoE switch below patch panel

☐  Mount UCG Ultra in rack (below switch or shelf-mount)

☐  Connect power to USW-48-PoE and UCG Ultra

☐  Run and terminate Cat6a cables from WAP locations to patch panel

☐  Connect patch cables from patch panel ports to corresponding switch ports

☐  Label all patch panel ports clearly (location + device)

☐  Install U7-Lite WAPs at designated ceiling/wall locations

☐  Connect WAP Cat6a cables to patch panel

☐  Apply cable management (velcro ties, service loops, neat routing)

☐  Verify all cable terminations with cable tester

☐  Take photo of completed rack build

Phase 2 — UCG Ultra Initial Setup (Phone App)

☐  Connect internet cable from building’s internet switch to UCG Ultra Port 5 (WAN)

☐  Power on UCG Ultra and wait for status LED to go solid white

☐  Open UniFi app on phone (download from App Store / Play Store if needed)

☐  Tap “Set Up New Device” — app should detect UCG Ultra via Bluetooth

☐  Create a new site (name it for the customer, e.g. “Snap Fitness Belmont”)

☐  Complete initial setup wizard (set admin credentials, timezone)

☐  Verify internet connectivity on the UCG Ultra dashboard

☐  Note down the UCG Ultra serial number in the Site Details table above

Phase 3 — Console Configuration (unifi.ui.com)

☐  Open a laptop/PC browser and navigate to unifi.ui.com

☐  Log in with your Ubiquiti account and select the site

☐  Navigate to Settings → System

☐  Update UniFi OS firmware to latest version

☐  Update Network application firmware to latest version

☐  Update any other control plane components if updates are available

☐  Restart UCG Ultra if prompted after firmware updates

☐  Verify all firmware versions are current after restart

Phase 4 — Network Configuration

☐  Navigate to Settings → Networks

☐  Edit the Default network:

☐  Change gateway/subnet to 192.168.10.1/24

☐  Set DHCP range: 192.168.10.100 – 192.168.10.254

☐  Set DNS: Primary 1.1.1.1, Secondary 8.8.8.8

☐  Create VLAN 20 — Equipment / IoT (192.168.20.0/24)

☐  Enable DHCP: 192.168.20.100 – 192.168.20.254

☐  Create VLAN 30 — Guest (192.168.30.0/24)

☐  Enable DHCP: 192.168.30.100 – 192.168.30.254

☐  Verify network changes have applied (may require device restart)

Phase 5 — WiFi Network Setup

☐  Navigate to Settings → WiFi

☐  Create [Site Name] Staff WiFi:

☐  Security: WPA3/WPA2 Personal

☐  Assign to VLAN 10 (Staff / Management)

☐  Set strong password — record in WiFi table above

☐  Create [Site Name] Cardio WiFi:

☐  Security: WPA2 Personal

☐  Assign to VLAN 20 (Equipment / IoT)

☐  Set password — record in WiFi table above

☐  Create [Site Name] Guests WiFi:

☐  Security: WPA2 Personal

☐  Assign to VLAN 30 (Guest)

☐  Enable client isolation

☐  Set bandwidth limits (e.g. 10 Mbps down / 5 Mbps up per client)

☐  Set password — record in WiFi table above

☐  Configure radio settings:

☐  2.4 GHz channel width: 20 MHz

☐  5 GHz channel width: 40–80 MHz

☐  Enable band steering (prefer 5 GHz)

☐  Set transmit power appropriate for venue size

Phase 6 — Device Adoption & Configuration

☐  Navigate to UniFi Devices in console

☐  Adopt USW-48-PoE switch (click Adopt if pending)

☐  Wait for switch to fully provision and update firmware

☐  Adopt each U7-Lite WAP

☐  Wait for all WAPs to fully provision and update firmware

☐  Configure switch port profiles:

☐  Assign staff/management ports to VLAN 10 (All)

☐  Assign equipment ports to VLAN 20

☐  WAP ports: set to All (trunk) so they can broadcast all SSIDs

☐  Assign static IP addresses to all UniFi devices:

☐  UCG Ultra: 192.168.10.1 (gateway — already set)

☐  USW-48-PoE: 192.168.10.2

☐  U7-Lite WAP 1: 192.168.10.3

☐  U7-Lite WAP 2: 192.168.10.4

☐  Record all MAC addresses in the Device table above

Phase 7 — Security & Optimisation

☐  Navigate to Settings → Security

☐  Enable Threat Management (IDS/IPS) on UCG Ultra

☐  Set sensitivity level to balanced (adjust based on site needs)

☐  Configure firewall rules:

☐  Block Guest VLAN (30) → Staff VLAN (10)

☐  Block Guest VLAN (30) → Equipment VLAN (20)

☐  Block Equipment VLAN (20) → Staff VLAN (10)

☐  Allow Staff VLAN (10) → All (management access)

☐  Enable DPI (Deep Packet Inspection) for traffic analytics

☐  Verify Remote Access is enabled (Settings → System → Cloud Access)

☐  Set auto-backup schedule:

☐  Navigate to Settings → System → Backups

☐  Enable automatic backups (weekly recommended)

☐  Set device LED brightness (dim or off for client-facing WAPs if required)

Phase 8 — Testing & Verification

☐  Connect a device to [Site] Staff WiFi — verify internet access

☐  Connect a device to [Site] Cardio WiFi — verify internet access

☐  Connect a device to [Site] Guests WiFi — verify internet access

☐  Verify VLAN isolation (Guest device cannot ping 192.168.10.1)

☐  Run speed test on each WiFi network — record results:

☐  Staff: _______ Mbps down / _______ Mbps up

☐  Cardio: _______ Mbps down / _______ Mbps up

☐  Guests: _______ Mbps down / _______ Mbps up

☐  Verify all switch ports are active and in correct VLAN

☐  Walk-test WAP coverage — confirm signal in all required areas

☐  Verify all static IPs are correctly assigned (Devices page)

☐  Test remote access via unifi.ui.com from phone (mobile data, not site WiFi)

☐  Verify all firmware versions are up to date

☐  Check UCG Ultra dashboard — all devices showing “Connected”

Phase 9 — Handover & Documentation

☐  Complete all tables on pages 1–2 of this document

☐  Take final photos of completed rack (front and rear)

☐  Record UCG Ultra serial number and firmware versions

☐  Document any site-specific notes or deviations below

☐  Provide site manager with WiFi credentials (Staff and Guest networks)

☐  Brief site manager on basic UniFi app access (optional)

☐  Obtain sign-off from site manager

Site-Specific Notes

Sign-Off

| Technician Name |  |
| --- | --- |
| Technician Signature |  |
| Date Completed |  |
| Site Manager Name |  |
| Site Manager Signature |  |
| Date Signed Off |  |

— End of Procedure —